1967.5 Datsun 1600

Steve Jacobson - work 665-1912, home 262-886-2186

Objectives (in order of importance):

- Reliability

- Safety

- Speed

Stuff I know about the car

- Isky cam, don’t know grind, trying to locate box

- Replaced SUs, have originals

- When we picked it up measured 210psi on all but #2 or #3 which was 190ish, but it was pretty much flooded, so could have been a bad read.

- Running premium unleaded pump gas

- Was last raced in ’04, old guy, probably not real aggressive, had old Bridgestone RE71 street tires.

To Do’s

- Door bars driver and passenger side

- Harness bar at correct height

- Weld/fix crack in firewall by master cylinder

- Fuel system

  - Move fuel pump to rear (old pump died after a couple days, bought a new Facet)

  - Install regulator and gauge

  - Check fuel cell

    - Smells of gas sometimes, seems to be on passenger side though, not where the lines are??

    - Foam OK?

    - Mounting: better, lower if possible, still needs street-able ground clearance

  - Other plumbing suggestions

- Tune carbs/needles/time/check advance - get it running right

  - Check valve clearance. Seems like valve train makes a lot of noise? I set clearance to stock spec’d .017 (hot), but I’m not the best measurer.

  - To get it running so far, I cleaned out old gas, replaced the yellow piston springs with red ones from the old carbs, “fixed” stuck choke on front carb, and put in the extra set of M39 needles.  (had M70 on front, M30 on rear).  ATF is current damper oil. Cleaned and lubed distributor. Still seems like fuel nozzles may be sticky.

- Electrical

  - Separate switches for ignition, fuel

- Safety check for track use

  - Frame welds

  - Any suggested reinforcement points?

  - Bearings/joints/grease

- ANY suggestions for improvements that can keep it street-able yet still run vintage (in the true vintage weekend warrior/drive it to the track sense)

  - Gauges

  - Velocity stacks/air cleaner(s)